# Propeller C Compiler

# User’s Manual

By Mike Christle

Table of Contents

Introduction	1

TERMS OF USE: MIT License	1

Build Process	2

Command Line Options	2

Directives	2

Error Reporting	2

Language Syntax	3

Comments	3

Data Types	3

Constants	3

Predefined Constants	3

Assembly Language Variable Names	4

Variable Declaration	4

Expressions Operators	5

Logical Operators	6

If Statements	7

For Loops	8

While Loops	9

Switch	10

Functions	11

Built In Functions	12

Math Functions	13

Waits	14

Global Data Access	15

Field Access	15

Parity	15

Miscellaneous Functions	16

BlinkLED Example Program	17

BNF Grammar	19

# Introduction

Does the world need another compiler? Probably not. However, I took a course in compiler design a couple of years ago and found the subject absolutely fascinating. Since then I have had a lot of fun working on this little project. If anyone else finds it useful, well that’s good to.

PropC is a command line compiler that outputs Parallax Propeller assembly code. The source language is based on C. Rather than trying to be C compliant; my priority was to allow efficient access to all the unique features of the Propeller. That’s why I added many functions and expressions that are simple wrappers around Propeller assembly instructions.

I recently started using the Parallax SimpleIDE. So I added an assembler that outputs a text file that can be included in the main C program.

# TERMS OF USE: MIT License

Copyright Mike Christle 2016

Permission is hereby granted, free of charge, to any person obtaining a copy of this software and associated documentation files (the "Software"), to deal in the Software without restriction, including without limitation the rights to use, copy, modify, merge, publish, distribute, sublicense, and/or sell copies of the Software, and to

permit persons to whom the Software is furnished to do so, subject to the following conditions:

The above copyright notice and this permission notice shall be included in all copies or substantial portions of the Software.

THE SOFTWARE IS PROVIDED "AS IS", WITHOUT WARRANTY OF ANY KIND, EXPRESS OR IMPLIED, INCLUDING BUT NOT LIMITED TO THE WARRANTIES OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE AND NONINFRINGEMENT. IN NO EVENT SHALL THE AUTHORS OR COPYRIGHT HOLDERS BE LIABLE FOR ANY CLAIM, DAMAGES OR OTHER LIABILITY, WHETHER IN AN ACTION OF CONTRACT, TORT OR OTHERWISE, ARISING FROM, OUT OF OR IN CONNECTION WITH THE SOFTWARE OR THE USE OR OTHER DEALINGS IN THE SOFTWARE.

# Build Process

The .c file is your PropC source. The .tmp file is created and used by the compiler; it contains the source for all included source files. The .a file contains the Propeller assembly source. The .a file can be assembled with the Propeller Tool, or passed to the PropC assembler. The .hex file contains the object code as C syntax hexadecimal constants. Use the #header and #footer directives to make this file an initialized integer array. The .lst file is an optional list file.

Optionally, a binary .bin file can be output instead of the .hex file. The binary file can be read into a Spin file using the Spin FILE command.

The first function encountered will become the main function regardless of the name. This function should loop forever.

PropC is not case sensitive. For example NAME = Name = name. This applies to all symbols and keywords.

## Command Line Options

-l	Output assembly list file.

-b	Output binary file instead of hex file.

-sa	Skip assembler stage.

-sc	Skip compiler stage, pass file directly to assembler.

-so	Skip optimizer, useful if object code is in doubt.

## Directives

#include “filename”	Include the named file.

#define label value	Define constants

#header “text”		Text is written to the top of the .hex file.

#footer “text”		Text is written to the bottom of the .hex file.

## Error Reporting

PropC will stop on the first error and output a (hopefully useful) error message and the source line number. The line number refers to the .tmp file which is a composite of all included source files. If no files are included then this is the same as the .c file.

# Language Syntax

All of the examples are snippets from .a files of test cases. That’s why the C code appears as assembly comments.

## Comments

PropC supports line comments, //. It does not support block comments, /* */.

## Data Types

Int UInt		32 bit signed and unsigned integers.

Real		32 bit fixed point number, with 16 bits on either side of the decimal point.

## Constants

Constants are specified similar to C, they start with a digit and the default is decimal. The 0x, 0o, and 0b prefixes specify hexadecimal, octal, and binary formats respectively. If the constant contains a period it is interpreted as a real data type, otherwise it is interpreted as an integer. Constants may include underscores which are ignored.

## Predefined Constants

The following constants are always defined. Type AINT means int or uint.

| Name | Value | Type | Description |
| --- | --- | --- | --- |
| true | 1 | AINT | TRUE |
| false | 0 | AINT | FALSE |
| odd | 1 | AINT | Used with parity function. |
| even | 0 | AINT | Used with parity function. |
| dira |  | UINT | Propeller register. |
| dirb |  | UINT | Propeller register. |
| ina |  | UINT | Propeller register. |
| inb |  | UINT | Propeller register. |
| outa |  | UINT | Propeller register. |
| outb |  | UINT | Propeller register. |
| par |  | UINT | Propeller register. |
| cnt |  | UINT | Propeller register. |
| ctra |  | UINT | Propeller register. |
| ctrb |  | UINT | Propeller register. |
| frqa |  | UINT | Propeller register. |
| frqb |  | UINT | Propeller register. |
| phsa |  | UINT | Propeller register. |
| phsb |  | UINT | Propeller register. |
| vcfg |  | UINT | Propeller register. |
| vscl |  | UINT | Propeller register. |

## Assembly Language Variable Names

Function names and global variables have the same name in the assembly code output. Function variable names are constructed by appending the function name, an underscore, and the variable name. For example a variable x declared in the main function will get the name main_x. Temporary variables are constructed by appending the function name, two underscores, and an integer, for example main__0.

## Variable Declaration

Variables are declared as in C. They can be initialized with a constant value of the same data type.

' int i1, i2 = -4; MOV main_i2, CONST_M_4 ' uint u1, u2 = 8; MOV main_u2, #8 ' real r1, r2 = 4.4; MOV main_r2, CONST_4_4 CONST_M_4 LONG -4 CONST_4_4 LONG 288358 main_i1 RES 1 main_i2 RES 1 main_u1 RES 1 main_u2 RES 1 main_r1 RES 1 main_r2 RES 1

## Expressions Operators

Expressions can be constructed using these operators. They are listed in order of precedence.

| Operator | Assignment | Description |
| --- | --- | --- |
| () |  | Parentheses. |
| + - ~ ! |  | Unary plus, minus, bitwise not, logical not. |
| * / % | *= /= %/ | Multiply, divide, modulo. |
| + - | += -= | Addition, subtraction. |
| << >> <- -> | <<= >>= <-= ->= | Shift left, shift right, rotate left, rotate right. |
| & &~ | &= &~= | Bitwise and, bitwise and not. |
| ^ | ^= | Bitwise exclusive or. |
| ¦ | ¦= | Bitwise or. |
| <# #> |  | Max, min. |

### Rotate

Rotates work just like shifts. Rotates have the same level of precedence as shifts.

' a = b <- c; MOV main_a, main_b ROL main_a, main_c ' a = b -> 5; MOV main_a, main_b ROR main_a, #5 ' a <-= 4; ROL main_a, #4 ' a ->= b; ROR main_a, main_b

### AndNot

' uint pin_mask; ' ' // Set a pin HI ' outa ¦= pin_mask; OR outa, main_pin_mask ' // Set a pin LO ' outa &~= pin_mask; ANDN outa, main_pin_mask

### Limits

Limits get the lowest precedence level, so they work best when added to the end of an expression. Limits can be applied to expressions of any type.

' int a, b, c; ' a = b + c #> 10 <# 100; MOV main_a, main_b ADDS main_a, main_c MINS main_a, #10 MAXS main_a, #100

## Logical Operators

Logical expressions can be constructed using these operators. They are listed in order of precedence.

| Operator | Description |
| --- | --- |
| == != < > <= >= LOCKSET LOCKCLR PARITY | Compare two expressions, or compare one expression to 0. Set and test a lock. Clear and test a lock. Test parity of an expression. |
| && | Logical and. |
| ¦¦ | Logical or. |

### Locks

| Function | Input Type | Output Type |
| --- | --- | --- |
| locknew | Uint |  |
| lockset | Uint | Uint |
| lockclr | Uint | Uint |
| lockret | Uint |  |

' uint lock, data_ready, data_ptr; ' ' locknew(lock); LOCKNEW main_lock ' ' // Block until lock is free ' while (lockset(lock) == true); :L1 LOCKSET main_lock WC IF_C JMP #:L1 ' // ... ' lockclr(lock); LOCKCLR main_lock WC ' ' // Proceed if data is ready and lock is free ' if (data_ready && lockset(lock) == false) CMP main_data_ready, #0 WZ IF_Z JMP #:L5 LOCKSET main_lock WC IF_C JMP #:L5 ' { ' // ... ' lockclr(lock); LOCKCLR main_lock WC :L5 ' } ' ' lockret(lock); LOCKRET main_lock

## If Statements

The syntax of the if statements is the same as C.

' int a, b, c; ' ' if (a == 3 ¦¦ b == 4 && c == 5) nop; CMPS main_a, #3 WZ IF_Z JMP #:L142 CMPS main_b, #4 WZ IF_NZ JMP #:L145 CMPS main_c, #5 WZ IF_NZ JMP #:L145 :L142 NOP :L145 ' ' if (a == b) CMPS main_a, main_b WZ IF_NZ JMP #:L154 ' { ' a = 9; MOV main_a, #9 JMP #:L155 ' } ' else ' { :L154 ' a = 6; MOV main_a, #6 :L155 ' }

## For Loops

The syntax of for loops is the same as C, except that the test and increment clauses are optional. The abbreviated form takes advantage of the DJNZ instruction. The break and continue statements work as expected.

' int i, j, k; ' for (i = j + 10) MOV main_i, main_j ADDS main_i, #10 ' k += 4; :L1 ADDS main_k, #4 DJNZ main_i, #:L1 ' ' for (i = 0; i < 10; i += 1) MOV main_i, #0 :L3 CMPS main_i, #10 WZ, WC IF_NC JMP #:L6 ' { ' if (i == 3) continue; CMPS main_i, #3 WZ IF_Z JMP #:L3 ' if (i == 7) break; CMPS main_i, #7 WZ IF_Z JMP #:L6 ' } ADDS main_i, #1 JMP #:L3 :L6

## While Loops

The syntax for the while and do while loops is the same as in C.

' int i, j; ' while (true) nop; :L1 NOP JMP #:L1 ' ' while (i < 40) :L4 CMPS main_i, #40 WZ, WC IF_NC JMP #:L6 ' { ' if (i == 10) continue; CMPS main_i, #10 WZ IF_Z JMP #:L4 ' if (i == 20) break; CMPS main_i, #20 WZ IF_NZ JMP #:L4 :L6 ' } ' ' do :L13 ' { ' if (i == 10) continue; CMPS main_i, #10 WZ IF_Z JMP #:L13 ' if (i == 20) break; CMPS main_i, #20 WZ IF_Z JMP #:L14 ' } ' while (i < 40); CMPS main_i, #40 WZ, WC IF_C JMP #:L13 :L14

## Switch

The syntax for the switch statement is the same as in C. The switch variable must be an integer type. The case values must be integer constants.

' int i, j; ' uint m; ' switch (i) ' { ' case 1: j += 3; break; CMPS main_i, #1 WZ IF_NZ JMP #:L2 ADDS main_j, #3 JMP #:L1 :L2 ' case 2: j -= 5; break; CMPS main_i, #2 WZ IF_NZ JMP #:L3 SUBS main_j, #5 JMP #:L1 :L3 ' default: j = 0; break; MOV main_j, #0 ' } :L1 ' ' switch (m) ' { ' case 1: j += 3; break; CMP main_m, #1 WZ IF_NZ JMP #:L5 ADDS main_j, #3 JMP #:L4 :L5 ' case 2: j -= 5; break; CMP main_m, #2 WZ IF_NZ JMP #:L6 SUBS main_j, #5 JMP #:L4 :L6 ' default: j = 0; break; MOV main_j, #0 ' } :L4

## Functions

The syntax for function declarations and calls is the same as in C.

' int g1; ' void main() ' { main ' int i; ' func1(); CALL #func1 ' i = func2(1, 5); MOV func2_a, #1 MOV func2_b, #5 CALL #func2 MOV main_i, func2_ ' } main_RET RET '--------------------------------------------------------- ' void func1() ' { func1 ' g1 += 3; ADDS g1, #3 ' } func1_RET RET '--------------------------------------------------------- ' int func2(int a, int b) ' { func2 ' return a + b; MOV func2_, func2_a ADDS func2_, func2_b ' } func2_RET RET '---------------------------------------------------------

## Built In Functions

The following functions, as well as multiply, divide and modulus, are implemented by subroutines that are included as needed. Parameters are passed to these functions via the variables math_p1 and math_p2. Results are passed back via the variables math_r1 and math_r2. The multiply, divide and modulus operations take 32 bit parameters and return 32 bit results. For the trig functions a full circle contains 512.0 degrees.

| Function | Input Type | Output Type |
| --- | --- | --- |
| sin | Real | Real |
| cos | Real | Real |
| tan | Real | Real |
| asin | Real | Real |
| acos | Real | Real |
| sqrt | Real | Real |
| isqrt | Int, Uint | Int, Uint |
| log | Any | Int |
| exp | Int | Int |

' int i, j; ' real m, n; ' i *= j; MOV math_p1, main_i MOV math_p2, main_j CALL #IMultiply MOV main_i, math_r1 ' i /= j; MOV math_p1, main_i MOV math_p2, main_j CALL #SDivide MOV main_i, math_r1 ' i %= j; MOV math_p1, main_i MOV math_p2, main_j CALL #SDivide MOV main_i, math_r2 ' m = sin(n); MOV math_p1, main_n CALL #Sin MOV main_m, math_r1

## Math Functions

| Function | Input Type | Output Type |
| --- | --- | --- |
| itor | Int | Real |
| rtoi | Real | Int |
| floor | Real | Real |
| ceil | Real | Real |
| trunc | Real | Real |
| round | Real | Real |
| fract | Real | Real |
| abs | Any | Same as input |
| absn | Any | Same as input |

' real m, n; ' i = rtoi(m); MOV main_i, main_m SAR main_i, #16 ' m = itor(i); MOV main_m, main_i SHL main_m, #16 ' m = floor(n); MOV main_m, main_n ANDN main_m, math_real_mask ' m = ceil(n); MOV main_m, main_n ADD main_m, math_real_mask ANDN main_m, math_real_mask ' m = trunc(n); MOV main_m, main_n ABS main_m, main_m ANDN main_m, math_real_mask NEGC main_m, main_m ' m = round(n); MOV main_m, main_n ADD main_m, math_half ANDN main_m, math_real_mask ' m = fract(n); MOV main_m, main_n ABS main_m, main_m AND main_m, math_real_mask NEGC main_m, main_m ' m = abs(n); ABS main_m, main_n ' m = absn(n); ABSNEG main_m, main_n math_real_mask LONG 65535

## Waits

These functions provide access to the Propellers four wait instructions.

' uint a, one_msec, mask; ' ' waitcnt(cnt + one_msec); MOV main__0, cnt ADD main__0, main_one_msec WAITCNT main__0, #0 ' waitcnt(a, one_msec); WAITCNT main_a, main_one_msec ' ' waitpeq(a, 4); WAITPEQ main_a, #4 ' waitpeq(a, mask); WAITPEQ main_a, main_mask ' ' waitpne(a, 8); WAITPNE main_a, #8 ' waitpne(a, mask); WAITPNE main_a, main_mask ' ' waitvid(a, 9); WAITVID main_a, #9 ' waitvid(a, mask); WAITVID main_a, main_mask

## Global Data Access

Global data is modelled as four arrays: GBYTE, GWORD and GLONG. The index for these arrays must be int or uint type. Any type can be read or written.

' uint uval, index; ' int ival; ' real rval; ' ' uval = GBYTE[index]; RDBYTE main_uval, main_index ' ival = GWORD[index]; RDWORD main_ival, main_index ' rval = GLONG[index]; RDLONG main_rval, main_index ' ' GBYTE[index] = uval; WRBYTE main_uval, main_index ' GWORD[index] = ival; WRWORD main_ival, main_index ' GLONG[index] = rval; WRLONG main_rval, main_index

## Field Access

The Inst, Src and Dest fields of a register can be accessed using the .I, .S and .D qualifiers on an assignment.

' uint apin, bpin; ' CTRA.S = apin; MOVS ctra, main_apin ' CTRA.D = bpin; MOVD ctra, main_bpin ' CTRA.I = 0b0_00100_000; MOVI ctra, #32

## Parity

This function will return the parity of a value. The constants ODD and EVEN are predefined for testing parity.

' uint i, j, mask; ' i = parity(mask, j); TEST main_mask, main_j WC SUBX main_i, main_i ' ' if (parity(mask, ina) == ODD) nop; TEST main_mask, ina WC IF_NC JMP #:L17 NOP :L17

## Miscellaneous Functions

These were included for completeness sake. Nop is useful for adding a 4 clock cycle delay.

' nop; NOP ' cmpsub(ival, 100); CMPSUB main_ival, #100 ' rev(ival, 16); REV main_ival, #16

# BlinkLED Example Program

This is the main program that compiles in SimpleIDE. It includes the cog code from the file BlinkLED_cog.hex.

// File BlinkLED.c #include "simpletools.h" #include "BlinkLED_cog.hex" typedef struct { volatile int pin_mask; volatile int half_period; int cog; } blink_t; int main() { int cog; blink_t *device; device = (void *) malloc(sizeof(blink_t)); device->pin_mask = 1; device->half_period = CLKFREQ >> 3; cog = 1 + cognew((void*)cog_code, (void *)device); while(1); }

This is the main program in Spin that runs in Propeller Tool. It includes the cog code from the file BlinkLED_cog.bin. The cog code must be compiled with the –b option.

CON _clkmode = xtal1 + pll16x _xinfreq = 5_000_000 VAR long Cog, PinMask, HalfPeriod PUB MainRoutine PinMask := 1 HalfPeriod := 20_000_000 Cog := cognew(@CogCode, @PinMask) repeat DAT CogCode file "BlinkLED_cog.bin"

This is the code to be compiled by PropC which runs in the cog.

// File BlinkLED_cog.c #header "int cog_code[] = {" #footer "};" uint half_period; uint pin_mask; uint wait_cntr; void main() { uint ptr; ptr = par; pin_mask = GLONG[ptr]; ptr += 4; half_period = GLONG[ptr]; dira = pin_mask; wait_cntr = cnt + half_period; while (true) { outa ^= pin_mask; waitcnt(wait_cntr, half_period); } }

This is the file BlinkLED_cog.hex, which is included into the SimpleIDE file.

int cog_code[] = { 0xA0BC1DF0,0x08BC180E,0x80FC1C04,0x08BC160E, 0xA0BFEC0C,0xA0BC1BF1,0x80BC1A0B,0x6CBFE80C, 0xF8BC1A0B,0x5C7C0007,0x5C7C0000};

# BNF Grammar

| <abc> | Rule name |
| --- | --- |
| ::= | Is defined by |
| ¦ | Or |
| [] | Optional |
| {} | Repeat one or more times |
| {}* | Repeat zero or more times |
| () | Group |
| ‘,’ | Literal symbol |
| ABC | Keyword |
| ; | Rule terminator |

<program> ::= { <data-decl-stmt> | <func-decl> }* ;

<func-decl> ::= <return-data-type> <identifier>

'(' [ <func-parms> ] ')' <func-body> ;

<func-parms> ::= <data-type> <identifier>

{ ',' <data-type> <identifier> }* ;

<return-data-type> ::= <data-type> | VOID ;

<func-body> ::= '{' { <data-decl-stmt> }* { <statement> } '}' ;

<data-decl-stmt> ::= <data-type> <data-decl> { ',' <data-decl> }* ';' ;

<data-decl> := <identifier> [ '=' [ '+' | '-' ] <constant> ] ;

<data-type> ::= INT | UINT | REAL ;

<statement> ::= <assignment-stmt>

| <function-stmt>

| <dot-assign-stmt>

| <if-stmt>

| <while-stmt>

| <do-while-stmt>

| <for-stmt>

| <switch-stmt>

| <break-stmt>

| <continue-stmt>

| <return-stmt>

| <compound-stmt>

| <global-array-stmt>

| <wait-count-stmt>

| <wait-stmt>

| <lock-stmt>

| <built-in-op-stmt>

| <nop-stmt>

| ';' ;

<assignment-stmt> ::= <assignment> ';' ;

<assignment> ::= <identifier> <assignment-op> <expr>

| <identifier> '=' [TRUE | FALSE] ;

<assignment-op> ::= '=' | '+=' | '-=' | '*=' | '/=' | '%='

| '|=' | '&=' | '^=' | '&~='

| '<<=' | '>>=' | '<-=' | '->=' ;

<dot-assign-stmt> ::= <identifier> ( '.I' | '.D' | '.S' )

'=' <expr> ';' ;

<global-array-stmt> ::= <global-array> '=' <expr> ';' ;

<global-array> ::= ( GBYTE | GWORD | GLONG ) '[' <expr> ']' ;

<if-stmt> ::= IF '(' <logical-or-expr> ')' <statement>

[ ELSE <statement> ] ;

<while-stmt> ::= WHILE '(' <logical-or-expr> ')' <statement> ;

<do-while-stmt> ::= DO <statement>

WHILE '(' <logical-or-expr> ')' ';' ;

<for-stmt> ::= FOR '(' <for-init>

[ ';' <logical-or-expr> ';' <for-incr> ] ')'

<statement> ;

<for-init> ::= <identifier> '=' <expr> ;

<for-incr> ::= <identifier> <assignment-op> <expr> ;

<switch-stmt> ::= SWITCH '(' <expr> ')'

'{' { <case-clause> }* [<default-clause>] '}' ;

<case-clause> ::= CASE <integer-constant> ':' { <statement> }* BREAK ';' ;

<default-clause> ::= DEFAULT ':' { <statement> }* BREAK ';' ;

<break-stmt> ::= BREAK ';' ;

<continue-stmt> ::= CONTINUE ';' ;

<return-stmt> ::= RETURN [<expr>] ';' ;

<function-stmt> ::= <function-call> ';' ;

<function-call> ::= <identifier>

'(' [ <expr> { ',' <expr>} }* ] ')' ;

<wait-count-stmt> ::= WAITCNT '(' <identifier> ',' <expr> ')' ';'

| WAITCNT '(' <expr> ')' ';' ;

<wait-stmt> ::= ( WAITPEQ | WAITPNE | WAITVID )

'(' <expr> ',' <expr> ')' ';' ;

<lock-stmt> ::= ( LOCKNEW | LOCKSET | LOCKCLR | LOCKRET )

'(' <IDENTIFIER> ')' ';' ;

<built-in-op-stmt> ::= ( CMPSUB | REV )

'(' <identifier> ',' <expr> ')' ';' ;

<compound-stmt> ::= '{' { <statement> } '}' ;

<nop-stmt> ::= NOP ';' ;

<logical-or-expr> ::= <logical-and-expr>

| <logical-or-expr '||' <logical-and-expr> ;

<logical-and-expr> ::= <relational-expr>

| <logical-and-expr '&&' <relational-expr> ;

<relational-expr> ::= <expr> '==' <expr>

| <expr> '!=' <expr>

| <expr> '<' <expr>

| <expr> '>' <expr>

| <expr> '<=' <expr>

| <expr> '>=' <expr>

| <lock-expr>

| <parity-expr> ;

<lock-expr> ::= ( LOCKSET | LOCKCLR ) '(' <identifier> ')'

( '==' | '!=' } ( TRUE | FALSE ) ;

<parity-expr> ::= <parity-function> ( '==' | '!=' } ( ODD | EVEN ) ;

<expr> ::= <inclusive-or-expr>

| <expr> '#>' <inclusive-or-expr>

| <expr> '<#' <inclusive-or-expr> ;

<inclusive-or-expr> ::= <exclusive-or-expr>

| <inclusive-or-expr> '|' <exclusive-or-expr> ;

<exclusive-or-expr> ::= <and-expr>

| <exclusive-or-expr> '^' <and-expr> ;

<and-expr> ::= <shift-expr>

| <and-expr> '&' <shift-expr>

| <and-expr> '&~' <shift-expr> ;

<shift-expr> ::= <additive-expr>

| <shift-expr> '<<' <additive-expr>

| <shift-expr> '>>' <additive-expr>

| <shift-expr> '->' <additive-expr>

| <shift-expr> '<-' <additive-expr> ;

<additive-expr> ::= <multiplicative-expr>

| <additive-expr> '+' <multiplicative-expr>

| <additive-expr> '-' <multiplicative-expr> ;

<multiplicative-expr> ::= <unary-expr>

| <multiplicative-expr> '*' <unary-expr>

| <multiplicative-expr> '/' <unary-expr>

| <multiplicative-expr> '%' <unary-expr> ;

<unary-expr> ::= <primary-expr>

| ! <primary-expr>

| ~ <primary-expr>

| - <primary-expr>

| + <primary-expr> ;

<primary-expr> ::= <identifier>

| <function-call>

| <global-array>

| <built-in-function>

| <parity-function>

| <built-in-operation>

| <integer-constant>

| <real-constant>

| '(' <expr> ')' ;

<built-in-function> ::= ( SIN | COS | TAN | ASIN | ACOS | LOG | EXP

| SQRT | ISQRT ) '(' <expr> ')' ;

<built-in-operation> ::= ( ABS | ABSN | ITOR | RTOI | CEIL | FLOOR

| TRUNC | ROUND | FRACT ) '(' <expr> ')' ;

<parity-function> ::= PARITY '(' <expr> ',' <expr> ')' ;

<identifier> ::= <letter> { <letter> | <digit> | '_' }* ;

<letter> ::= (a-zA-Z) ;

<digit> ::= (0-9) ;

<constant> ::= <integer-constant> | <real-constant> ;

<real-constant> ::= <dec-constant> '.' [ <dec-constant> ] ;

<integer-constant> ::= <dec-constant> | <hex-constant>

| <oct-constant> | <bin-constant> ;

<dec-constant> ::= { 0-9 | _ } ;

<hex-constant> ::= '0x' { 0-9 | A-F | _ } ;

<oct-constant> ::= '0o' { 0-7 | _ } ;

<bin-constant> ::= '0b' { 0 | 1 | _ } ;